UNIVERSITY OF MANAGEMENT AND TECHNOLOGY

School of System and Technology — Department of Computer Science

COMPLEX COMPUTING PROBLEM (CCP) REPORT

COMPUTER NETWORKS (CN-S2026)

PROBLEM 7 — PCAP FORENSICS & ATTACK ISOLATION

Instructor: Sir Tahir Mehmood  |  Section: V16  |  Date: June 4, 2026

GROUP MEMBERS

Ariba Waseem (F2024266905) — Git & Deployment Lead

Atiqa Kamran (F2024266914) — Technical & Topology Specialist

Ayesha Khan (F2024266918) — Documentation Specialist

Ifrah Tariq (F2024266939) — Risk & Resource Analyst

# 1. Week 3 Activities Completed

- Deployed and verified ACL (Access Control List) to block botnet C2 nodes at the gateway router.

- Configured BLOCK_BOTNET_C2 extended ACL targeting attacker IPs 219.243.27.65 and 71.181.6.40 on TCP Port 23.

- Applied ACL inbound on GigabitEthernet0/0 interface — match counters confirmed active blocking.

- Enforced Layer 2 port security on Raspberry Pi's switchport (Fa0/10) — MAC address locked to b827.ebc7.e9c9.

- Violation mode set to Shutdown — any unauthorized MAC triggers port disable automatically.

- Validated all configurations via show commands and documented verification logs.

- Compiled full final report integrating Week 1, Week 2, and Week 3 findings.

# 2. Milestones Achieved

- ACL deployed and active — both attacker IPs blocked at gateway with confirmed packet hit counts.

- Layer 2 port security enforced — Raspberry Pi MAC address locked; unauthorized access triggers automatic shutdown.

- Multi-layer defense complete — ACL (Layer 3) + Port Security (Layer 2) + VLAN Isolation + SSH replacement fully operational.

- Final report compiled — all forensic findings, configurations, and evidence consolidated.

# 3. ACL Configuration — Botnet C2 Blocking

Problem 7 required isolation of the attacker C2 servers identified in the PCAP forensic analysis. An extended ACL was deployed to drop all Telnet traffic from the two threat IPs before it reaches any internal device.

ACL Definition:

ip access-list extended BLOCK_BOTNET_C2

deny tcp host 219.243.27.65 any eq 23

deny tcp host 71.181.6.40 any eq 23

permit ip any any

Applied to Gateway Interface:

interface GigabitEthernet0/0

ip access-group BLOCK_BOTNET_C2 in

Verification Log — Packet Hit Counts:

| Seq | ACL Rule | Action | Match Count |
| --- | --- | --- | --- |
| 10 | deny tcp host 219.243.27.65 any eq 23 | DENY (Blocked) | 1,452 matches |
| 20 | deny tcp host 71.181.6.40 any eq 23 | DENY (Blocked) | 894 matches |
| 30 | permit ip any any | PERMIT (Allowed) | 52,341 matches |

Conclusion: Both C2 attacker IPs are actively and continuously blocked at the gateway. Legitimate traffic (52,341 matches) passes unaffected.

# 4. Layer 2 Port Security — Switchport Enforcement

To prevent MAC spoofing or unauthorized device replacement on the Raspberry Pi's switch port, Layer 2 port security was enforced with a static MAC address binding and a violation shutdown policy.

Port Security Configuration:

interface FastEthernet0/10

description Secure Link to Raspberry Pi

switchport mode access

switchport access vlan 20

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address b827.ebc7.e9c9

switchport port-security violation shutdown

Verification Output:

| Parameter | Value |
| --- | --- |
| Port Security | Enabled |
| Port Status | Secure-up |
| Violation Mode | Shutdown |
| Maximum MAC Addresses | 1 |
| Total MAC Addresses | 1 |
| Configured MAC Address | b827.ebc7.e9c9 (Raspberry Pi) |

Conclusion: Port Fa0/10 is locked to the legitimate Raspberry Pi MAC address. Any unknown device plugged in will immediately trigger a port shutdown, preventing hardware-level intrusion.

# 5. Complete Multi-Layer Defense Summary

| Layer | Control Deployed | Target Threat | Status |
| --- | --- | --- | --- |
| Layer 7 | Telnet Disabled / SSH Enforced | Unencrypted remote access | ✓ Verified |
| Layer 3 | ACL BLOCK_BOTNET_C2 | C2 IPs 219.243.27.65 & 71.181.6.40 | ✓ Active (2,346 blocked) |
| Layer 2 | VLAN 20 IoT Isolation | Lateral movement / broadcast | ✓ Verified |
| Layer 2 | Port Security — MAC Lock | MAC spoofing / rogue device | ✓ Secure-up |

# 6. Week 3 Challenges & Resolutions

| Challenge | Resolution |
| --- | --- |
| ACL permit statement needed to avoid blocking legitimate traffic | Added 'permit ip any any' as final rule — verified via match counters |
| Port security violation mode selection | Chose 'Shutdown' (strictest) over 'Restrict' — matches threat severity of botnet intrusion |
| Confirming ACL is applied inbound vs outbound | Applied inbound on Gi0/0 — filters attacker packets before they enter the router |

# 7. Next Steps / Final Submission

- Final report (CN_Final_Report.docx) compiled integrating all three weeks of work.

- All evidence screenshots, Wireshark captures, and configuration logs attached.

- Peer review completed within group before final submission.

- Submission to Sir Tahir Mehmood — CN-S2026 via designated portal.

8. Github link:

https://github.com/aribawaseemsiddiqui08/PCAP-Forensics-Problem7.git

Submitted to: Sir Tahir Mehmood  |  UMT — School of System and Technology  |  Computer Networks CN-S2026