ACCESS CONTROL LIST (ACL) TO BLOCK BOTNET COMMAND NODES

PROBLEM 7: PCAP FORENSICS & ATTACK ISOLATION

1. DEFINING THE EXTENDED ACL PACKET FILTERS ---

ip access-list extended BLOCK_BOTNET_C2

deny tcp host 219.243.27.65 any eq 23

deny tcp host 71.181.6.40 any eq 23

permit ip any any

2. APPLYING THE ACL FILTER TO THE GATEWAY INTERFACE ---

interface GigabitEthernet0/0

ip access-group BLOCK_BOTNET_C2 in

3. VERIFICATION LOG SHOWING ATTACKER HIT COUNTS ---

Router# show access-lists BLOCK_BOTNET_C2

Extended IP access list BLOCK_BOTNET_C2

10 deny tcp host 219.243.27.65 any eq 23 (1452 matches)

20 deny tcp host 71.181.6.40 any eq 23 (894 matches)

30 permit ip any any (52341 matches)